Three-Part Specifications - Copyright 2025 - Nixalite( of America Inc

Pigeon Spike Composite Bird Spikes
Includes specifications for: Pigeon Spike Composite Bird Spikes.
The Pigeon Spike Composite Bird Spikes are an economical deterrent for pigeons and seagulls that use a combination (or composite) of flat tipped stainless-steel wires and a UV stabilized polycarbonate base. They are not intended to repel smaller or larger bird species and are not for use in moderate to heavy populations of any bird species.
List Pigeon Spike Composite Bird Spike 3-part specifications in the following locations;

Master Format 2004 – 10 81  00 - Pest Control Devices
Master Format 2004 – 10 81  13 – Bird Control Devices
PART 1 - GENERAL

1.1 - SYSTEM DESCRIPTION:

A. Pigeon Spike Composite Bird Spikes:
A physical bird spike deterrent designed to hinder pigeons and seagulls from landing on architectural surfaces. Not for smaller or larger bird species or heavy infestations of any bird species.
B. Fastening:
Pigeon Spike Composite Bird Spikes can be fastened to all types of installation surface materials with the optional mechanical fasteners or adhesive.
C. Surface Cleaning System: disinfectants and deodorizers to neutralize potentially hazardous bird and/or animal wastes and prepare the surface for installation.
1.2 - QUALITY ASSURANCE

A.
Installer to obtain, review and understand Pigeon Spike Composite Bird Spike planning guides, estimating worksheets and installation instructions. Installer must be completely familiar with the proper installation procedures for the Pigeon Spike Composite bird spike.

B.
Installer must obtain and record accurate and complete dimensions for each surface specified for Pigeon Spike Composite Bird Spike installation. Surface dimensions are what drive bird spike totals.
1.3 - SUBMITTALS

A.
Pigeon Spike Composite Bird Spike estimating and installation guidelines.

B.
Sample of Pigeon Spike Composite Bird Spike - not less than 2” (5.1 cm) in length.

1.4 – STORAGE & HANDLING
A. Provide storage and/or protection to keep Pigeon Spike Composite Bird Spike shipping boxes dry, clean and undamaged. Do not stack or place other packaging or objects on the bird spike shipping boxes.

B. Keep Pigeon Spike Composite Bird Spikes in the original packaging until needed for installation.

PART 2 - PRODUCTS

2.1 – ACCEPTABLE MANUFACTURER

A. Nixalite® of America Inc 
1025 16th Avenue, East Moline, Illinois 61244; U.S.A.
P: 309.755.8771 - E: planning@nixalite.com or sales@nixalite.com    www.nixalite.com 

B. ABC Advanced Bird Control – A division of Nixalite® of America Inc.
PO Box 727, East Moline, Illinois 61244, U.S.A.
P: 309.755.4708   E: info@abcbirdcontrol.com     www.abcbirdcontrol.com 

2.2 –PIGEON SPIKE COMPOSITE BIRD SPIKE:
A. Wires: Flat-tipped stainless-steel.
B. Base: UV stabilized polycarbonate resin.
C. Strip Lengths: 13” (33.3 cm) long.

D. Width & Height: 5” high (12.7 cm), 6” wide (15.2 cm).

E. Points per Inch: 20 flat-tipped wire points per piece.
F. Application: Installed on all types of surfaces to deter nuisance level roosting problems caused by pigeons and seagulls.
G.
Finish: Natural finish on stainless steel wire. Opaque polycarbonate base.
2.3 –FASTENING
A.
Pigeon Spike Composite Bird Spikes can be be fastened with Nixalite’s optional mechanical mounting hardware or with the adhesive.
2.4 – SURFACE SANITIZING AND DEODORIZING
A.
Nixalite of America Inc offers several different broad spectrum surface sanitizers and deodorizers to help protect your people from the hazards of pest bird waste. Contact Nixalite for recommendations on the proper sanitizing product for your application.
B.
Safety Equipment: Nixalite offers personal protection equipment (PPE) to protect personnel from the hazards related to pest bird and animal waste materials. Contact Nixalite for details.
PART 3 - EXECUTION

3.1 - INSPECTION

A.
Visually inspect all installation surfaces. Make sure all surfaces are clean, dry and free from debris or other conditions that could impede the workflow of this section.

B.
Notify architect of detrimental conditions. Do not proceed until these conditions have been corrected.

3.2 - PREPARATION

A.
Field Measurements: Verify the dimensions for each surface specified for Pigeon Spike Composite Bird Spike installation. Use suppled planning and estimating guides to verify that sufficient quantities of Pigeon Spike Composite Bird Spikes will be installed on each surface specified for bird control.

B.
Make sure all installation surface finishing requirements have been accomplished before installing Pigeon Spike Composite. Bird spikes are to be the last items installed on each specified surface. Do not apply any surface coating or treatment (paint, sealer, etc.) over the installed spikes or mounting hardware.

C.
Remove or relocate all plants, foliage or foreign objects that overhang the installation surfaces. Note all conditions that could adversely affect the installation and performance of the bird spike installation.

3.3 – SURFACE CLEANING

A.
All surfaces to be clean, dry and free of obstructions before the Pigeon Spike Composite bird spikes are installed.

B.
If Bird Waste Is Present: 
Treat, neutralize and safely remove all bird waste from installation surfaces. Installer must follow all municipal, state and federal regulations regarding the proper removal and disposal of bird droppings and waste materials such as nests and dead birds.

C.
Use Nixalite’s surface cleaning products to neutralize any bird droppings, nests and related waste materials that may be present. Strictly follow treatment instructions provided with Nixalite’s surface cleaning products.

D.
Use Nixalite cleaning products to help prevent disease transmittal when working around surfaces contaminated with bird droppings.

3.4 - INSTALLATION

A.
Install Pigeon Spike Composite Bird Spikes in accordance with manufacturer’s estimating and installation guidelines. Protect all surfaces.
B.
Protect the entire surface, not just the outside edges. Adjust length of spike base by snapping the composite base strip apart at the Quick Part Seams (notches on the base strip between the wires).
C.
Wires of Pigeon Spike Composite to be flush with or overhanging the outside edge of all surfaces. At open end of a surface, overhang the first spike on the base at least 1/2” (1.2 cm). Use the alignment notch in the base strip to align the spikes that meet end-to-end.
D.
Do not allow gaps between spikes that meet end-to-end. Do not allow gaps between the spikes and other objects such as walls or beams.
E.
Fasten Pigeon Spike Composite to the surface with the optional mounting hardware or adhesive (purchased separately).
3.5 – ADJUSTMENTS / CLEANING

A.
Remove debris and waste materials from project site. Inspect finished installation. Make any adjustments needed to conform to the Pigeon Spike Composite’s spacing and installation guidelines.

B.
Pigeon Spike Composite Bird Spikes are a physical and passive surface deterrent. The stainless-steel wires deter pigeons and seagulls from landing on the surfaces on which they are installed. Periodic inspections are recommended to keep the bird spikes clean and free of any debris.

Nixalite of America Inc, 1025 16th Ave., East Moline, IL. 61244
P: 309.755.8771   E: planning@nixalite.com   www.nixalite.com
Copyright 2025 – Nixalite( of America Inc
Nixalite( is a registered trademark of Nixalite( of America Inc.  Printed in the USA.

